Preventive Maintenance Checklist + Break/Fix SOP

Version: 0.1  |  Date: Feb 18, 2026

SS NII Mobile X-ray Inspection Vans

# Part 1 - Preventive Maintenance (PM) Checklist

Use this checklist as a baseline. Align frequencies and procedures to the specific van OEM manuals and local safety requirements.

## A. Job information

| Field | Value |
| --- | --- |
| Site | [ ] |
| Asset ID(s) | [ ] |
| Technician(s) | [ ] |
| Date | [ ] |
| Work order / ticket no. | [ ] |
| Downtime window | [ ] |

## B. Safety pre-checks (always)

Confirm controlled-area boundaries, signage, and traffic control are in place before enabling X-rays.

Verify emergency stop function and interlocks prior to scanning operations.

Use radiation survey meter per procedure; ensure calibration is current.

Apply lockout/tagout (LOTO) where required for electrical/mechanical service steps.

Record any safety defects immediately and stop work if necessary.

## C. Operator daily checks (sample)

| Task | Frequency | Result | Notes |
| --- | --- | --- | --- |
| Visual inspection of van exterior, cables, and warning devices | Daily | [Pass/Fail] | [ ] |
| Verify warning lights/sirens function during pre-op check | Daily | [Pass/Fail] | [ ] |
| Confirm no error codes on workstation; verify storage space | Daily | [Pass/Fail] | [ ] |
| Radiation controlled area setup (cones/barriers/signs) | Daily | [Pass/Fail] | [ ] |
| Survey meter operational check (battery/self-test) | Daily | [Pass/Fail] | [ ] |

## D. Technician PM tasks

| Task | Frequency | Completed (Y/N) | Notes / evidence |
| --- | --- | --- | --- |
| Check safety interlocks and e-stop circuits | Monthly | [ ] | [ ] |
| Inspect HV connections and cable condition (per OEM safe procedure) | Quarterly | [ ] | [ ] |
| Verify warning light/siren circuits and indicators | Quarterly | [ ] | [ ] |
| Clean/inspect detector housing and protective covers | Quarterly | [ ] | [ ] |
| Inspect cooling system (fans/filters), clean as required | Quarterly | [ ] | [ ] |
| Workstation health: logs review, disk health, backups | Quarterly | [ ] | [ ] |
| Software/firmware version review; apply approved updates | Semi-annual | [ ] | [ ] |
| Image quality baseline check using test object | Quarterly | [ ] | [ ] |
| Radiation leakage survey at defined points | Quarterly or after repair | [ ] | [ ] |
| Verify survey meter calibration status; replace batteries | Quarterly | [ ] | [ ] |
| Grounding continuity check (if applicable) | Annual | [ ] | [ ] |
| Review incident log and recurring faults; create RCA actions | Annual | [ ] | [ ] |

## E. Closeout

Remove tools, restore site condition, and confirm controlled-area items are removed/secured as required.

Record parts used and update Asset Register (last PM, next due, software version).

Provide PM report to customer POC within [2] business days.

# Part 2 - Break/Fix Standard Operating Procedure (SOP)

## 1. Purpose and scope

Define a consistent workflow for incident intake, triage, remote diagnostics, onsite repair, safety checks, and service restoration for SS NII mobile X-ray inspection vans.

## 2. Definitions

| Term | Working definition |
| --- | --- |
| Ticket | A logged support request with a unique identifier. |
| Triage | Initial assessment to assign severity, gather logs, and define next actions. |
| Service restored | Scanning function returned to normal operations or an agreed workaround is implemented. |

## 3. Workflow

| Step | Activities |
| --- | --- |
| Step 1 - Intake | Log ticket; capture site, asset ID, symptoms, time of occurrence, safety concerns; assign initial severity. |
| Step 2 - Safety screen | If interlock failure, warning device failure, or suspected radiation safety issue: stop scanning immediately and escalate to RSO/management. |
| Step 3 - Remote diagnostics | Collect logs, error codes, screenshots; run standard checks; guide operator through safe resets; confirm controlled-area setup before any scan tests. |
| Step 4 - Dispatch decision | If not recoverable remotely, confirm onsite access, permits, escorts, and required parts; schedule dispatch per SLA. |
| Step 5 - Onsite repair | Perform LOTO as required; replace/repair components; document parts used; update configuration if needed. |
| Step 6 - Post-repair verification | Run functional tests, interlock checks, warning devices test, and image quality check; perform radiation survey if required by procedure. |
| Step 7 - Closeout | Communicate results; document RCA and preventive actions; update asset and parts records; obtain customer confirmation. |

## 4. Minimum data to collect (per incident)

Asset ID(s), serial numbers, software/firmware versions

Date/time of failure and operational context (shift, weather, load)

Error codes, logs, screenshots, and recent changes (updates/repairs)

Safety conditions (controlled-area status, interlocks/warnings)

Parts replaced and configuration changes

Verification evidence (checklist references, survey results if applicable)

## 5. Escalation

| Trigger | Escalate to | Timeframe |
| --- | --- | --- |
| Suspected safety hazard or overexposure | RSO + Service Delivery Manager | Immediate |
| Repeat failure within 30 days | Engineering / OEM liaison | Within 1 business day |
| Parts backorder threatens SLA | Supply chain lead + Account Manager | Within 1 business day |

## 6. Record templates (optional)

Use the attached fields in your ticketing system or a printed service report:

| Field | Entry |
| --- | --- |
| Ticket No. | [ ] |
| Severity | [S1/S2/S3/S4] |
| Customer POC | [ ] |
| Problem description | [ ] |
| Actions taken (remote) | [ ] |
| Actions taken (onsite) | [ ] |
| Parts used | [ ] |
| Verification performed | [ ] |
| Service restored time | [ ] |
| Follow-up / RCA actions | [ ] |