Radiation Safety Program (X-ray Devices) + Training Record Templates

Version: 0.1  |  Date: Feb 18, 2026

For SS NII Mobile X-ray Inspection Vans

Template for commercial discussion only. Not legal advice. X-ray device regulations differ by jurisdiction. Align this program with local requirements and OEM instructions.

## 1. Purpose

Define minimum controls for safe operation, servicing, and training associated with ionizing radiation generated by SS NII mobile X-ray inspection vans.

## 2. Scope

Applies to operation, commissioning, maintenance, and troubleshooting activities involving X-ray generation and associated safety systems. Applies to employees, contractors, and visitors participating in or supporting scanning operations.

## 3. Roles and responsibilities

| Role | Responsibilities |
| --- | --- |
| Radiation Safety Officer (RSO) | Owns the program, approves procedures, reviews incidents, maintains records, and coordinates regulatory interactions. |
| Operations Supervisor | Ensures only trained/authorized operators perform scanning and that controlled areas are established each shift. |
| Operators | Follow SOPs, maintain controlled-area controls, perform daily checks, and immediately report hazards. |
| Service Technicians | Follow service safety steps, LOTO, and post-repair verification including surveys when required. |
| Customer Representative | Ensures site readiness, traffic control, and local permit compliance as applicable. |

## 4. ALARA and controlled-area practices

Apply ALARA principles: minimize time near the beam, maximize distance, and use shielding/barriers as applicable.

Establish controlled area boundaries before enabling X-rays (barriers, cones, signage).

Ensure warning lights/sirens are functional and visible/audible to approaching traffic.

Never bypass interlocks or safety devices except under an approved service procedure and only when X-rays are disabled.

## 5. Equipment safety features (baseline)

| Feature | Expectation |
| --- | --- |
| Emergency stop | Stops X-ray generation and places system in safe state. |
| Access interlocks | Prevent X-ray enablement when access doors/panels are open (as designed). |
| Warning indicators | Visual/audible warnings when X-rays are about to be enabled and while enabled. |
| Key control / authorization | Restricts operation to authorized personnel. |
| Survey meter availability | A calibrated survey meter is available for commissioning, periodic checks, and post-repair verification where required. |

## 6. Dosimetry (as required)

Dosimetry requirements vary by jurisdiction and job role. Where required, issue personal dosimeters to operators and technicians. Track issuance/return and review reports for anomalies.

## 7. Surveys and measurements

Baseline radiation survey during commissioning (defined measurement points and operating conditions).

Periodic survey at a defined frequency (e.g., quarterly) or after repairs affecting shielding/interlocks.

Maintain survey meter calibration records and battery checks.

## 8. Training and competency

Minimum training topics (tailor to site and device):

Radiation fundamentals and X-ray device hazards

ALARA and controlled-area setup

System safety features (interlocks, warnings, e-stop)

Normal operating procedures and prohibited actions

Emergency procedures and incident reporting

Dosimetry use (if applicable) and survey meter awareness

Competency should be confirmed via observation, a short assessment, or supervisor sign-off.

## 9. Incidents and emergency response

Stop scanning and make the area safe. Use emergency stop as needed.

Notify site supervisor and RSO immediately for any suspected safety system failure, uncontrolled exposure, or injury.

Preserve logs and do not alter system state beyond safety actions unless directed by RSO/engineering.

Document incident details and corrective actions; report to authorities as required.

## 10. Records and retention

Training records (attendance, assessments, authorizations)

Survey and calibration logs

Incident reports and corrective actions

Maintenance and PM records related to safety systems

Dosimetry logs and reports (if applicable)

# Appendix A - Training Record Templates

## A1. Training attendance sheet

| Course | Site | Date | Instructor | Duration |
| --- | --- | --- | --- | --- |
| [Course title] | [ ] | [ ] | [ ] | [ ] |

| No. | Participant name | Role | Organization | Signature |
| --- | --- | --- | --- | --- |
| 1 | [ ] | [ ] | [ ] | [ ] |
| 2 | [ ] | [ ] | [ ] | [ ] |
| 3 | [ ] | [ ] | [ ] | [ ] |
| 4 | [ ] | [ ] | [ ] | [ ] |
| 5 | [ ] | [ ] | [ ] | [ ] |

## A2. Competency / authorization checklist (example)

| Skill item | Observed (Y/N) | Comments |
| --- | --- | --- |
| Sets up controlled area with correct boundaries/signage | [ ] | [ ] |
| Performs pre-operation safety checks (warnings, interlocks) | [ ] | [ ] |
| Demonstrates correct start/stop and emergency stop use | [ ] | [ ] |
| Understands prohibited actions (no bypassing interlocks) | [ ] | [ ] |
| Understands incident reporting procedure | [ ] | [ ] |

Authorized for operation: [Y/N]   Authorized by (name/title): [ ]   Date: [ ]

# Appendix B - Logs and forms

## B1. Dosimetry issue/return log (if applicable)

| Person | Dosimeter ID | Issue date | Return date | Notes |
| --- | --- | --- | --- | --- |
| [ ] | [ ] | [ ] | [ ] | [ ] |
| [ ] | [ ] | [ ] | [ ] | [ ] |
| [ ] | [ ] | [ ] | [ ] | [ ] |

## B2. Survey meter inventory + calibration log

| Meter model | Serial | Calibration due | Battery check date | Status |
| --- | --- | --- | --- | --- |
| [ ] | [ ] | [ ] | [ ] | [ ] |
| [ ] | [ ] | [ ] | [ ] | [ ] |

## B3. Radiation survey record (baseline / periodic)

| Site | Asset ID | Survey date | Surveyor | Operating condition |
| --- | --- | --- | --- | --- |
| [ ] | [ ] | [ ] | [ ] | [e.g., scan mode / kV / mA] |

Measurement points (attach sketch and record readings):

| Point ID | Location | Reading | Units | Notes |
| --- | --- | --- | --- | --- |
| 1 | [ ] | [ ] | [ ] | [ ] |
| 2 | [ ] | [ ] | [ ] | [ ] |
| 3 | [ ] | [ ] | [ ] | [ ] |
| 4 | [ ] | [ ] | [ ] | [ ] |

# Appendix C - Incident report form

| Field | Entry |
| --- | --- |
| Date/time | [ ] |
| Site | [ ] |
| Asset ID | [ ] |
| People involved / witnesses | [ ] |
| Incident description | [ ] |
| Immediate actions taken | [ ] |
| Safety system status (interlocks/warnings/e-stop) | [ ] |
| Survey performed (Y/N) and results | [ ] |
| Potential exposure (if any) | [ ] |
| Notifications made (RSO, management, authority) | [ ] |
| Corrective actions / preventive actions | [ ] |
| Reported by | [Name/title] |
| Approved by (RSO) | [Name/title] |